[image: image1.jpg]Peter PLUMBING LTD

Ph (04) 904 6429
JaCkson Mobile (027) 478 9880

Registered Craftsman Plumber www.peterjacksonplumbing.co.nz




The do's and don'ts for Septic tanks

Keep the treatment system "alive"

House hold cleaners, strong detergents and toxic chemicals can kill the good bacteria with in the treatment systems. To keep these bacteria alive;

DO - Use biodegradable detergents and cleaners.

Check your detergents and cleaners and see if they are suitable for use with a septic tank system.

Limit as much as possible the use of bleach-based products such as whiteners, nappy soakers and stain removers.

DON'T - Pour acids, pesticides, medicines, paint, thinners, engine oil or other materials which kill the bacteria into your septic tank. (These should be disposed of properly at you r local transfer station or landfill)

Pour grease or cooking oil into your septic tank; or

Use septic tank "cleaning chemicals"

Reduce the Solid Load

To reduce the sludge build up in your septic tank system;

DO - Scrape all your dishes to remove fat and food particles before washing; and

Shake sand and dirt from clothes before you wash them.

DON'T - Put sanitary pads, tampons, paper tissue, paper towels, disposable nappies, nappy liners, cigarettes of rags into the system;

use a kitchen waste disposal unit; or

put compostable vegetable matter into the treatment system unless it is specified by the manufacturer.

Reduce the Liquid Load

Small loads of effluent don't pass through the treatment system as quickly. This gives the solids in the effluent more time to settle out of the liquid rather than escaping out of the tank into the disposal system. Less effluent going into the system also means less effluent coming out. Therefore, the soils beneath the disposal system don't get as wet, providing a more effective final treatment for the effluent.

To reduce the amount of effluent going into the tank;

DO- Install water saving devices eg; dual flush toilet cisterns,

Take showers instead of baths; and

Fix leaking taps

DON'T - Wash clothes until you have a full load;

Do all the clothes washing on the same day. If you normally do more than two full loads of washing per week, try to spread it out over a few days.

Empty large volumes of water into the septic tank from Spa Pools; or

Allow stormwater to enter the septic tank, either from roof or surrounding land.

Protect the Septic Tank

DO - Restrict access to any areas of land that have been specially planted as part of the septic tank.

DON'T - Allow vehicle or stock access to areas where a septic tank has been installed.

Allow stormwater to enter the septic tank system, from the roof or surrounding land; or

Grow deep rooting trees or shrubs over the disposal system

We recommend

· Inspecting the system, vent caps in place, checking there is no sign of damage or leaks and access lids are accessible (not buried under 1m Lawn)

· Checking sludge and scum levels and record, ideally sludge levels should be no more  than half the total tank depth, or in a tow compartment tank, the sludge level should be below the partitioning wall.

· Inspect and clean the effluent filter, if there is no filter we recommend installing one as it will extend the life of the disposal fields.

· Check for any blockages in the inlet and outlet of the tank.

Peter Jackson

Certifying Plumber, Gasfitter, Drainlayer and ECCA Approved Solar Installer

